Part1: Web Browsing (DNS, TCP)

1. Find the first DNS request packet sent by the client.(Request for cse.nsysu.edu.tw)

You can find a record like below on Wireshark. And you can answer the question.

(1) Examine the Ethernet

a. What is the Ethernet address of the source and destination?

Source: HewlettP_4f:6b:41 (40:a8:f0:4f:6b:41)

Destination: JuniperN_73:14:01 (88:e0:f3:73:14:01)

b. What is the content of the type field in the Ethernet frame?

Type: IPv4 (0x0800)

(2) Examine the Internet Protocol

a. What is the IP address of the source and destination?

source: 140.117.171.179

destination: 140.117.11.1

b. What is the header length? What is the total packet length?

Header Length: 20 bytes

Total Length: 62 bytes

c. Identify the protocol type field. What is the number and type of the protocol in the payload?

Protocol: UDP (17)

(3) Examine the User Datagram Protocol

a. Identify the client ephemeral port number and the server well-known port number .

client:55438

Server:53

b. What type of application layer protocol is in the payload?

UDP

(4) Examine the Domain Name System (query)

a. What field indicates whether the message is a query or a response?

Domain Name System (query)

b. What is the query transaction ID?

Transaction ID: 0x8b98

c. Identify the fields that carry the type and class of the query.

Flags: 0x0100 Standard query

Questions: 1

Answer RRs: 0

Authority RRs: 0

Additional RRs: 0

2. Find the DNS response packet which is response to the DNS request packet from the above question.

You can find a record like below on Wireshark. And you can answer the question.

- Examine the Ethernet.

a. What is the Ethernet address of the source and destination?

Source: JuniperN_73:14:01 (88:e0:f3:73:14:01)

Destination: HewlettP_4f:6b:41 (40:a8:f0:4f:6b:41)

b. What is the content of the type field in the Ethernet frame?

Type: IPv4 (0x0800)

- Examine the Internet Protocol & Domain Name System (response)

a. What is the IP address of the source and destination?

Source: 140.117.11.1

Destination: 140.117.171.179

b. What is the header length? What is the total packet length? Is it longer than the query?

Header Length: 20 bytes (5)

Total Length: 182 bytes

Yes，response比query長

c. How many answers are provided in the response message? Compare the answers and their time-to-live values.

One，time to live:3

3. Find the first TCP packet sent by client. (The destination IP address is response from above question.) You can find three record like below on Wireshark. It’s TCP three-way handshake.

(1) Examine the Transmission Control Protocol.

- What are the ephemeral port number used by the client and the well-known port number used by the server?

client :50936

server:80

- What is the length of the TCP segment?

Len:0

- What is the initial sequence number for the segments from the client to the server?

Sequence number: 0    (relative sequence number)

- What is the initial window size?

Window size value: 8192

- What is the maximum segment size?

Maximum segment size: 1460 bytes

f. Find the hex character that contains the SYN flag bit

Flags: 0x002 (SYN)

Part 2 Probing the Internet (ICMP, PING, Traceroute)

1. Ping Captured.

(1) Find the first ICMP Echo Request packet.

a. First, examine the Internet Protocol. What is the Time-to-Live?

Time to live: 128

b. Next examine the Internet Control Message Protocol. What is the ICMP message type?

Type: 8 (Echo (ping) request)

c. What is the message identifier and sequence number?

Identifier (BE): 1 (0x0001)

Identifier (LE): 256 (0x0100)

Sequence number (BE): 1 (0x0001)

Sequence number (LE): 256 (0x0100)

(2) Find the first ICMP Echo Reply packet.

- Examine the Internet Protocol. What are the source and destination    addresses?

Source: 8.8.8.8

Destination: 140.117.171.179

b. Next, examine the Internet Control Message Protocol. What is the ICMP message type?

Type: 0 (Echo (ping) reply)

2. Traceroute Captured.

(1) Find the first ICMP Echo Request packet.

a. Examine the Internet Protocol. What are the source and destination addresses?

Src: 140.117.171.179, Dst: 8.8.8.8

- What are the protocol type and the Time-to-Live in the IP packet?

Time to live: 1

Protocol: ICMP (1)

- Next, examine the Internet Control Message Protocol. What is the ICMP message type? What are the message identifier and sequence number?

Identifier (BE): 1 (0x0001)

Identifier (LE): 256 (0x0100)

Sequence number (BE): 5 (0x0005)

Sequence number (LE): 1280 (0x0500)

(2) Find an ICMP Time-to-live exceeded packet.

- Examine the Internet Protocol. What are the source and destination addresses?

Source: 140.117.162.254

Destination: 140.117.171.179

- Next, examine the Internet Control Message Protocol. What is the ICMP message type?

Type: 11 (Time-to-live exceeded)

Part 3 Measuring Network Bandwidth

1. Measure the bandwidth for Transmission Control Protocol

Type “iperf3 -c 140.117.171.171 -t 10 -i 2”

2. Adjust the window size for Transmission Control Protocol. See what’s different.

Type “iperf3 -c 140.117.171.171 -w 2000 -t 10 -i 2”

3. Measure the bandwidth for User Datagram Protocol

Type “iperf3 -c 140.117.171.171 -u -t 10 -i 2”

4. Adjust the bandwidth for User Datagram Protocol.Measure the package lost rate or any else happened.

Type “iperf3 -c 140.117.171.171 -u -t 10 -i 2 -b 512G”